Install Required boards in Arduino IDE

Open Board Manager by navigating to the Tool>>Board

Search for Arduino SAMD Boards and install the Arduino SAMD Boards

Now search for 	Adafruit SAMD and install Adafruit SAMD Boards

Now if you are using Windows 7 or 8 you need to download and install the driver from the link.

https://github.com/adafruit/Adafruit_Windows_Drivers/releases

Install Required Libraries

Open Library Manager by navigating to the Sketch>> Include Library.

Search for the Adafruit_GFX and click install.

Search for the Adafruit_FT6206 and click install.

Search for the Adafruit_ILI9341 and click install.

Search for RTCLib.h

Search for TimeLib.h

Now go to Sketch >> Include Library >> ADD .ZIP library

Navigate the Library folder provided with this folder and select FlashStorage_SAMD.zip file.

Now restart Arduino IDE.

Connections

Connect LCD and RTC with Arduino Board.

Connections of RTC with Arduino.

| RTC Pins | Arduino Metro M4 Pins |
| --- | --- |
| VCC | 5v |
| GND | GND |
| SDA | SDA |
| SCL | SCL |

Upload the code in Arduino.

RTC will set its time when the first-time code is uploaded in Arduino.

If you want to delete old data from the memory there is a variable (MAGIC_NUMBER) in config.h file,

Change its value to random number between 100-200.